# Steps to configure your Deliberant CPE-2

These are the steps to configure the CPE firmware after it has been installed in the Deliberant CPE-2.  This configuration with establish a WISP Mode of operation that will be connected to a wireless router in the WAN port so wireless router can be connected to a wireless AP (like a RV park Wi-Fi).

Assumptions:

The assumption is that the CPE firmware has been installed in the CPE-2 and the Factory Defaults have been set.

The PC is directly connected with a CAT-5 cable to the CPE POE Power Injector.

The PC’s IP address has been set to 192.168.2.201 as described in The Steps to Change Your PC’s IP Address.

Configuration Steps

1.	Open a web browser like Internet Explorer or Forefox.

In the URL Address field of the browser, type 192.168.2.66.  This is the default address of the AP Firmware.

Hit the Enter Key.

2.	In the CPE firmware LOGIN screen:

Enter admin in the LOGIN field and admin01 in the PASSWORD field.  These are the default username and password,

Click the Login button.

3.	In the Status screen:

Normally you will be taken to the Status tab screen.

Click on the Configuration tab.

4.	On the Configuration – Network screen:

On the Configuration tab and the Network sub-tab: click on the Operating Mode .  Select Router for the pull-down menu.

5.	On the Configuration – Network screen:

Click on the Access type  and select DHCP Client from the pull-down menu.

6.	On the Configuration – Network screen:

Click on the Operating Mode  and select server from the pull-down menu.

7.	On the Configuration – Network screen:

In the IP address field, enter 192.168.nn.254 where nn is the loop address desired.  In this case, nn = 15.

In the IP address from field, change the entry to 192.168.nn.100 where nn = the desired loop address.  In the case nn = 15.

In the IP address to field, change the entry to 192.168.nn.199 where nn = the desired loop address.  In the case nn = 15.

In the DNS server 2 field, change the entry to 4.2.2.1.

Finally click the Save button.

8.	On the Configuration – Network screen:

A little clock will appear at the cursor while the save is in process.  Wait.

9.	On the Configuration – Network screen:

When the save is done, the “Configuration saved.” message will appear.

Click on the Management tab.

10.	On the System tab screen:

Click on the Reboot button.

11.	Reboot confirmation.

A box will pop up to confirm that you want to reboot.  Click on the Yes button.

12.	Please wait screen:

The indicated Please Wait bar will appear while the CPE is rebooting.  This takes a while, be patient.  Figure about 2 minutes.

13.	CPE firmware Login screen:

When this screen appears, you cannot login.  This is one of the last steps in the configuration change / reboot process.  The IP address for the CPE has changed and you will have to change the PC’s IP Address.

14.	In the Internet Protocol Version 4 screen:

Change the third field in the IP address filed to the same nn and was entered back in the CPE Configuration – Network screen;.

In this case, nn is 15.

Click the OK button and the other OK and Close buttons related to change your PC’s IP Address.

15.	In a Command window:

In Pinging You PC’s IP Address, all the steps are covered.

In the Command window: enter the IP Address you set in the CPE Configuration – Network screen, in this case, 192.168.15.254

Observe that the CPE  is responding to the Ping command.

16.	In your web browser window:

Enter the AP IP Address set in the CPE Configuration– Network screen.  In this case, 192.168.15.254.

17.	In the CPE firmware LOGIN screen:

Enter admin in the LOGIN field and admin01 in the PASSWORD field.  These are the default username and password,

Click the Login button.

18.	On the Status tab screen:

Click on the System tab.

19.	On the System tab screen:

Enter admin01 in the Old Password field.  Enter a Unique password in the New password field and the Verify password field.  Write this password down and save in in a place where you will be sure to remember.

Click on the Change password button.

20.	On the System tab screen:

After the clock cursor goes away, the “Password has been changed” message will appear.

Click on the Save button.

21.	On the System tab screen:

When the clock cursor goes away and the save is complete, the “Configuration saved.” message appears.

Click on the Reboot Button.

22.	Reboot confirmation.

A box will pop up to confirm that you want to reboot.  Click on the Yes button.

23.	Please wait screen:

The indicated Please Wait bar will appear while the CPE is rebooting.  This takes a while, be patient.  Figure about 2-4 minutes.

24.	On the System tab screen:

When the Please Wait bar goes away, you will need to Logout.  Click on the indicated Logout button.

25.	In the CPE firmware Login screen:

In the Username field, enter admin.

In the Password field, enter the Unique password entered in the CPE System tab screen.

t

26.	On the System tab screen:

Click on the Backup button.

27.	Select location for download window:

This is what FireFox will display.  Internet Explorer will have a similar display.

Navigate to the fold where you are keeping all the Deliberant files in the upper address field.

Change the File name to an appropriate name like WISP2.cfg that describes this saved configuration file as a CPE file.

Click on the Save button.

28.	On the System tab screen:

When the “Fire successfully downloaded.” message appears, you are done.

29.	On the Configuration tab – Wireless sub-tab screen:

The Antenna option selects the Internal or External antenna.