## Steps to Establish Connection to Wi-Fi Source:

1.  If your PC is wire connected to your router, go to step 2.  Connect your laptop to your router wire your wireless adapter.

2.  Open your web browser (e.g. Internet Explorer, FireFox, and Chrome)

Enter 192.168.15.254 in the indicated URL field.  Sometimes you need to add the http:// in front.  Hit the keyboard Enter key.

3.	On the CPE firmware Login screen:

Enter the usercode and password.

In your case, this would be admin and csp140.

4.	On the Configuration tab – Wireless sub-tab screen:

Normally you will be taken to the Configuration tab and the Wireless tab on that screen.

5.	On the Configuration tab – Wireless sub-tab screen:

Click on the indicated Scan button.  The cursor will change to a spinning clock.  Wait until the cursor return to normal.

If you see a red alert at the bottom of the screen then the connection timed-out, reselect the Scan button until a scan completes.

6.	On the Configuration tab – Wireless sub-tab screen:

Select the indicated dropdown box by clicking on the  on the end of the box.

7.	On the Configuration tab – Wireless sub-tab screen:

Select the SSID of the Wi-Fi source you want to connect to by clicking on the name.  The source is usually evident from the name.

8.	On the Configuration tab – Wireless sub-tab screen:

After you have selected the SSID, the Security mode for that Wi-Fi source will be set as indicated.  If the Security mode is Open System, there is no security key needed. Go to step 11.

9.	On the Configuration tab – Wireless sub-tab screen:

Click on the  at the end of the Security mode dropdown box.  From the key types offered, select one by clicking on it as indicated.  If you need a key, the Wi-Fi source supplier will usually indicate the type.  If the key is something like 10 or 26 numbers, then the key is probably a WEP 64 or WEP 256 key.  If the supplied key is something like “mountainviewrvpark” then the key is probably a WPA key.  When in doubt, ask the supplier.

10.	On the Configuration tab – Wireless sub-tab screen:

Here a WPA key is entered in the indicated Passphrase field.  Note the check mark under the field Show passphrase is set.  This shows the characters you type instead of “*****”.

11.	On the Configuration tab – Wireless sub-tab screen:

Once all the changes on this screen are done, click on the indicated Save button.  The cursor will change into a clock while saving the parameters. When the cursor returns to normal, a white box will appear at the bottom of the screen indicating that a Reboot is necessary for the parameters changes to take effect.

12.	On the System tab screen:

Click on the System tab at the top of the screen.  Then click on the indicated Reboot button on the System screen.

13.	Reboot confirmation.

A box will pop up to confirm that you want to reboot.  Click on the Yes button.

14.	Please wait screen:

The indicated Please Wait bar will appear while the CPE is rebooting.  This takes a while, be patient.

15.	On the System tab screen:

When the Please Wait bar goes away, proceed to check your connection.

16.	On the Status tab screen:

Click on the Status tab at the top of the screen.  On the Status screen, check that the ESSID is the one you selected (indicated at top of screen).

Check that the RSSI power level is reasonable as indicated at the bottom of the screen.  The greater the RSSI power the better your connect will run.(-60 is better then -40).

The Data Rate indicated at the top of the screen shows how good the connection is to the Wi-Fi source.

The Wireless IP address (indicated screen middle) shows the address that the Wi-Fi source has given the CPE.  Only issue would be if there was no IP address.

17.

Open another tab on your browser and try to connect to something on the Internet.  This tests the connection.  At this point, some Wi-Fi sources may send a screen that requires a logon to their system.  Enter the username and password you were given for this purpose.

18.	Done