# Steps to change from CPE to AP mode for the CPE-2

These are the steps to configure the CPE firmware after it has been installed in the Deliberant CPE-2.  This configuration with establish a WISP Mode of operation that will be connected to a wireless router in the WAN port so wireless router can be connected to a wireless AP (like a RV park Wi-Fi).

Assumptions:

The assumption is that the CPE firmware has been installed in the CPE-2.

There is an AP configuration file.

The PC is directly connected with a CAT-5 cable to the CPE POE Power Injector.

The PC’s IP address has been set to 192.168.nn.201 where nn is the CPE working IP address node as described in Changing Your PC’s IP Address.

The PC is directly connected with a CAT-5 cable to the CPE POE Power Injector.

Configuration Steps

1.	In the Internet browser window:

Enter the IP Address of CPE LAN address.  In this case it is 192.168.15.254.

2.	In the CPE firmware Login screen:

Enter admin in the LOGIN field and the Unique password in the PASSWORD field.

Click the Login button.

3.	On the Status tab screen:

Click on the System tab.

4.	On the System tab screen:

Click on the Upload firmware button.

5.	In the Select file to upload window:

In the Look in field, navigate to where the Deliberant files are located. In this case CPE2.

Locate the AP firmware; in this case it is DLB-AP_2i.v5.26.24841.090917.170627.img.

Click on the Open button.

6. 	On the System tab screen:

After a short upload period, the screen will refresh with the Upgrade button present.  This indicates that the new firmware has been copied to the CPE-2 and is ready to be installed.

Click on the Upgrade button.

The firmware change will takes a few minutes, as many as 7.

7.	On the AP firmware LOGIN screen

Enter admin in the LOGIN field and the Unique password in the PASSWORD field.

Click the Login button.

8.	On the STATISTICS – System Information screen:

Click on the SYSTEM button.

9.	On the SYSTEM – Account screen:

Click on the Maintenance button.

10.	On the SYSTEM – Maintenance screen:

Click on the Reset button.

Since the address of the AP has been changed, you now need to change the IP Address of you PC.

All the steps have been covered in Changing You PC’s IP Address.

11.	In the Internet Protocol Version 4 (TCP/IPv4) Properties window:

Select the Use the following IP address: radio button.  Then enter the IP address that you need.  The first two sets of three digits will be 192 168.2.201

The Subnet mask: should automatically be filled with “255.255.255.0”  If not, enter the digits.

When you are done, click on the OK and Close buttons to close the open windows.

One thing to do to insure that the Reboot process has finished, use The Steps to Check Your PC’s IP Address and PING the IP Address 192.168.2.66 until you get responses.

12.	In your Internet browser window:

Enter the IP Address of the Default CPE LAN address.  In this case it is 192.168.2.66.

13.	On the AP firmware LOGIN screen:

Enter admin in the LOGIN field and admin01 in the PASSWORD field.  These are the default username and password,

Click the Login button.

14.	On the STATISTICS – System Information screen:

Click on the CONFIGURATION button.

15.	On the CONFIGURATION – Network screen:

Click on the Expert button:

16.	On the CONFIGURATION – Expert screen:

Click on the Browse button.

17.	In the File Upload window:

Navigate to the folder with the Deliberant files in the upper address field.

Select the AP configuration file.  In this case it is AP.cfg.

18.	On the CONFIGURATION – Expert screen:

Click on the Upload button.

19.	On the CONFIGURATION – Expert screen:

After a short period, the screen will refresh with the “Configuration has been uploaded” message displayed.

Click on the Reboot button.

20. 	On the AP firmware LOGIN screen:

Enter admin in the LOGIN field and the Unique password in the PASSWORD field.

Click the Login button.

You are done.