# Steps to change from CPE to AP mode for the CPE-2

These are the steps to configure the CPE firmware after it has been installed in the Deliberant CPE-2.  This configuration with establish a WISP Mode of operation that will be connected to a wireless router in the WAN port so wireless router can be connected to a wireless AP (like a RV park Wi-Fi).

Assumptions:

The assumption is that the CPE firmware has been installed in the CPE-2.

There is a CPE configuration file.

The PC is directly connected with a CAT-5 cable to the CPE POE Power Injector.

The PC’s IP address has been set to 192.168.nn.201where nn is the CPE working IP address node as described in Changing Your PC’s IP Address.

The PC is directly connected with a CAT-5 cable to the CPE POE Power Injector.

Configuration Steps

1.	In the Internet browser window:

Enter the IP Address of CPE LAN address.  In this case it is 192.168.1.254.

2.	In the AP firmware LOGIN screen:

Enter admin in the LOGIN field and the Unique password in the PASSWORD field.

Click the Login button.

3.	On the STATISTICS – System Information screen:

Click on the SYSTEM button.

4.	On the SYSTEM – Account screen:

Click on the Maintenance button.

5.	On the SYSTEM – Maintenance screen:

Click on the Browse button.

6.	In the Choose File to Upload window:

In the Look in field, navigate to where the Deliberant files are located. In this case CPE2.

Locate the AP firmware; in this case it is DLB-AP_2i.v5.26.24841.090917.170627.img.

Click on the Open button.

7.	On the SYSTEM – Maintenance screen:

After a short period the screen will refresh with the selected file.

Click on the Upload button.

8.	On the SYSTEM – Maintenance screen:

After the firmware file is uploaded to the CPE-2, the Firmware Upgrade screen will appear

Click on the Upgrade button.

9.	On the Firmware wait screen:

As the screen says, wait.

10.	In the Internet browser window:

When this error appears, the firmware upgrade has occurred and the CPE is rebooting.

11.	In a Command window:

A trick to know when to proceed is to open a Command window, see The Steps to Check Your PC IP Address.

Ping the IP Address of the AP, in this case 192.168.1.254 with a –t which causes a continuous ping.

12.	In a Command window:

When the AP starts to respond to the ping you can continue.

13.	In the Internet browser window:

You have to enter the IP Address of the AP, in this case 192.168.1.254.

14.	On the CPE firmware Login screen:

You will note the screens have changed.

Enter admin in the Username field and the Unique password in the Password field.

Click the Login button.

15.	On the Status tab screen:

Click on the System tab.

16.	On the System tab screen:

Click on the Reset button.

17.	On the Reboot Confirm screen:

Click on the Yes button.

18.	On the Please Wait screen:

Do as it says, wait.

Since the CPE IP Address has been changed because of the Factory Defaults, it is time to change the PC’s address as described in The Steps to Change Your PC’s IP Address.

19.	In the Internet Protocol Version 4 (TCP/IPv4) Properties window:

Select the Use the following IP address: radio button.  Then enter the IP address that you need which is 192 168.2.201

The Subnet mask: should automatically be filled with “255.255.255.0”  If not, enter the digits.

When you are done, click on the OK and Close buttons to close the open windows.

One thing to do to insure that the Reboot process has finished, use The Steps to Check Your PC’s IP Address and PING the IP Address 192.168.2.66 until you get responses.

20.	In the Internet browser window:

Enter the IP Address of the Default CPE LAN address.  In this case it is 192.168.2.66.

21.	In the CPE firmware Login screen:

Enter admin in the LOGIN field and admin01 in the PASSWORD field.  These are the default username and password,

Click the Login button.

22.	On the Status tab screen:

Click on the System tab.

23.	On the System tab screen:

Click on the Restore button.

24.	In the Select file to upload window:

In the Look in field, navigate to where the Deliberant files are located. In this case CPE2.

Locate the AP firmware; in this case it is WISP2.cfg.

Click on the Open button.

25.	On the System tab screen:

After the configuration file is uploaded, the “Device configuration was ssuccessfully restored” message will appear.

Click on the Reboot button.

26.	On the Reb0ot Confirm screen:

Click on the Yes button.

27.

Do as it says, wait.

Since the CPE IP Address has been changed because of the load of the configuration file, it is time to change the PC’s address as described in The Steps to Change Your PC’s IP Address.

28.	In the Internet Protocol Version 4 (TCP/IPv4) Properties window:

Select the Use the following IP address: radio button.  Then enter the IP address that you need which in this case is 192 168.15.201

The Subnet mask: should automatically be filled with “255.255.255.0”  If not, enter the digits.

When you are done, click on the OK and Close buttons to close the open windows.

One thing to do to insure that the Reboot process has finished, use The Steps to Check Your PC’s IP Address and PING the IP Address 192.168.15.254 until you get responses.

29.	In your Internet browser window:

Enter the IP Address of the Default CPE LAN address.  In this case it is 192.168.15.254.

30.	In the CPE firmware Login screen:

Enter admin in the LOGIN field and the Unique password in the PASSWORD field.

Click the Login button.

You are done.