# Configure AP Firmware in Bridge Mode

These are the steps to configure the AP firmware after it has been installed in the Deliberant CPE-2.  This configuration with establish a Bridge Mode of operation that will be connected to a wireless router as one of the LAN devices (not the WAN port) so that the wireless router signal can be broadcasted over a larger area.

Assumptions:

The assumption is that the AP firmware has been installed in the CPE-2 and the Factory Defaults have been set.

The PC is directly connected with a CAT-5 cable to the CPE POE Power Injector.

The PC’s IP address has been set to 192.168.2.201 as described in Changing Your PC’s IP Address.

## Configuration Steps.

1.	Open a web browser like Internet Explorer or Forefox.

In the URL Address field of the browser, type 192.168.2.66.  This is the default address of the AP Firmware.

Hit the Enter Key.

2. In the AP Firmware Login Window:

Enter admin in the LOGIN field and admin01 in the PASSWORD field.  These are the default username and password,

Click the LOGIN button.

3.	In the STATISTICS – System Information screen:

Click on the CONFIGURATION button.

4.	In the CONFIGURATION – Network screen:

Insure Operating Mode is set to Bridged Mode.

in the IP address field, enter the desired loop address. 192.168.nn.254.  The nn is the loop node number.

In the Default gateway field, enter the desird loop gateway.  192.168.nn.1 where nn is equal to the nn in the IP Address field.

When done, click on the Submit button.

5.	In the CONFIGURATION – Network screen:

Note the SAVE option has appeared but do not click on it yet.

Click on the Wireless Settings button.

6.	In the CONFIGURATION – Wireless Settings screen:

Click on the Edit button.

7.	In the CONFIGURATION – Wireless Settings for interface ath0 screen:

Change the SSID field from Deliberant to a name that you want your AP to appears in a wireless scan.  In this case, RedRover_AP was entered.

Then click on the OK button.

8.	In the CONFIGURATION – Wireless Settings screen:

Now click on the SAVE button.

9.	In the CONFIGURATION – Wireless Settings screen:

After a short delay, the screen will refresh with the REBOOT button appears.

Click on the REBOOT button.

10.	With the Device is rebooting, please wait display:

Do just that, wait.  This may take 2 to 4 minutes.

11.	When the The connection has timed out screen appears:

Since the address of the AP has been changed, you  now need to change the IP Address of you PC.

All the steps have been covered in Changing You PC’s IP Address.

12.	In the Internet Protocol Version 4 screen:

Change the third field in the IP address filed to the same nn and was entered back in the AP CONFIGURATION – Network screen;

In this case, nn is 1.

Click the OK button and the other OK and Close buttons related to change your PC’s IP Address.

13.  In a Command window:

In Pinging You PC’s IP Address, all the steps are covered.

In the Command window: enter the IP Address you set in the AP CONFIGURATION – Network screen, in this case, 192.168.1.254

Observe that the AP  is responding to the Ping command.

14.	In your web browser window:

Enter the AP IP Address set in the AP CONFIGURATION – Network screen.  In this case, 192.168.1.254.

15. In the AP firmware LOGIN screen:

Again enter the username and password, admin and admin01.

16.	In the STATISTICS –System Information screen:

Click on the SYSTEM button.

17.	In the SYSTEM – Account screen:

In  the Old password field, enter admin01.

In the New password and Verify password fields, enter a unique password.

18.	In the SYSTEM – Account screen:

Note the “Password has been successfully changed” banner.

Click on the SAVE Button.

19.	In the SYSTEM – Account screen:

After a short delay, the screen will refresh with a REBOOT button.

Click on the REBOOT button.

20.	With the Device is rebooting, please wait display:

Do just that, wait.  This may take 2 to 4 minutes

You will not receive “The connection has timed out” screen.  Just wait the 2 to 4 minutes and refresh you browser.

21.	In the AP firmware LOGIN screen:

Enter admin in the LOGIN field and the Unique password you entered in the SYSTEM – Account screen in the PASSWORD field.

Click on the LOGIN button.

22.	In the STATISTICS – System Information screen:

Click on the CONFIGURATION button.

23.	In the CONFIGURATION – Network screen.

Click on the Expert button.

24.	In the CONFIGURATION – Expert screen:

Click on the Download button.

24.	in the File Download window:

This window will depend on your Internet browser.  This is the Internet Explorer window.  FireFox will have a similar window,

Click on the Save button.

24.	In the Save As window:

This is what Internet Explorer will display.  FireFox will have a similar display.

Navigate to the fold where you are keeping all the Deliberant files in the upper address field.

Change the File name to an appropriate name like AP2.cfg that describes this saved configuration file as an AP file.

Click on the Save button.

25.	In the Download complete window:

This is the Internet Explorer result window.  FireFox may load the file to the Downloads folder.  In that case, open the Downloads folder and move the file, which will probably have the name, system.cfg, to the folder where all the Deliberant files are located.  Rename the file to something appropriate that indicates that this is an AP configuration file, like AP2.cfg.

26.	Completion.

You have configured the AP firmware and saved that configuration is case it needs to be reloaded.

As long as the configuration stays the same, the IP Address for the AP firmware will be the one entered in the CONFIGURATION – Network screen and the password for logging in will be the Unique password entered in the SYSTEM – Account screen.